| MINISTRY OF INDUSTRY AND TRADE No: 1701/QD-BCT | SOCIALIST REPUBLIC OF VIETNAM Independence - Freedom - Happiness Hanoi, August 29, 2022 |
| --- | --- |

### DECISION

Approving the planning adjustment of the Đắk Glun 3 Hydropower Project, Đắk Nông province

THE MINISTER OF INDUSTRY AND TRADE

- Pursuant to the Law on Construction;

- Pursuant to Resolution No. 751/2019/UBTVQH14 dated August 16, 2019, of the National Assembly Standing Committee explaining several articles of the Law on Planning;

- Pursuant to Decree No. 98/2017/ND-CP dated August 18, 2017, of the Government defining the functions, tasks, powers, and organizational structure of the Ministry of Industry and Trade;

- Based on the request of Đắk Glun Hydropower Joint Stock Company in Document No. 50/2021/CV dated March 25, 2021, regarding the adjustment of the planning for the Đắk Glun 3 hydropower project, Tuy Duc district, Đắk Nông province;

- At the proposal of the Director General of the Electricity and Renewable Energy Authority.

### DECIDES:

Article 1. Approve the planning adjustment for the Đắk Glun 3 Hydropower Project (previously approved by the People's Committee of Đắk Nông province in Decision No. 571/QD-UBND dated April 30, 2008) with the following main contents:

- Construction location: On Đắk Glun stream (a level I tributary of the Bé River), located in Đắk Ngo and Quảng Tâm communes, Tuy Đức district, Đắk Nông province.

- Project tasks and exploitation scheme: The main task is to generate electricity for the national grid. The exploitation scheme includes a main water-raising dam combined with a spillway on the Đắk Glun stream, and a secondary dam on the Đắk K'long stream to supplement water to the energy line via channels and pipelines.

- Key Technical Parameters:

| No. | Parameter | Unit | Approved (Decision 571) | Adjusted (This Decision) |
| --- | --- | --- | --- | --- |
| 1 | Total catchment area (Flv) | Km2 |  | 86.0 |
|  | - Main dam catchment area (Flvdc) | Km2 |  | 70.6 |
|  | - Secondary dam catchment area (Flvdp) | Km2 |  | 15.4 |
| 2 | Normal water level - Main dam (MNDBTdc) | m |  | 500 |
| 3 | Normal water level - Secondary dam (MNDBTdp) | m |  | 605 |
| 4 | Minimum tailwater level (MNHLmin) | m |  | 397.46 |
| 5 | Installed capacity (Nlm) | MW | 6.5 | 7.1 |
| 6 | Average annual energy output (E0) | 106 kWh |  | 24.6 |

Article 2. Following the adjustment, the project belongs to the National Small Hydropower Planning in Đắk Nông province and will be integrated into the National Electricity Development Plan. Investment must comply with regulations on construction, water resources, and forest protection.

Article 3. During the investment research phase, the People's Committee of Đắk Nông province shall direct the Investor and relevant agencies to:

- Update project parameters and approve the Basic Design as regulated.

- Assess and implement measures to mitigate environmental and social impacts, ensuring minimum downstream flow.

- Ensure construction quality management according to the Law on Construction.

Article 4. Implementation schedule: The project is scheduled to be put into operation during the 2021–2025 period.

Article 5. This Decision takes effect from the date of signing.

Article 6. The Chief of the Ministry Office, Director General of the Electricity and Renewable Energy Authority, Chairman of the People's Committee of Đắk Nông province, and the Project Investor are responsible for implementing this Decision.

FOR THE MINISTER VICE MINISTER (Signed and Sealed)

Đặng Hoàng An