### DAK GLUN 2 HYDROPOWER PROJECT

LOCATION: TUY DUC DISTRICT, DAK NONG PROVINCE STAGE: TECHNICAL DESIGN

### 1. GENERAL INTRODUCTION

- Overview: The Dak Glun 2 Hydropower Project is the second stage in five planned locations on the Dak Glun stream.

- Project Location: The dam and powerhouse are located in Quang Tam Commune, Tuy Duc District, Dak Nong Province.

- Objectives:

  - Main duty: Power generation with an installed capacity of 3.9MW.

  - Annual average energy output: 13.64 million kWh.

  - Reservoir useful capacity: 54,000 m3.

- Investment Form: B.O.O (Build – Own – Operate).

### 2. MAIN TECHNICAL SPECIFICATIONS

- Hydraulic Features:

  - Catchment area: 51.70m3.

  - Average annual discharge (Q0): 2.78m3/s.

  - Design flood discharge (P=1.5%): 652.00 m3/s.

  - Check flood discharge (P=0.5%): 909.00 m3/s.

- Reservoir Levels:

  - Normal Water Level (NWL): 628.10 m.

  - Dead Water Level (DWL): 625.50 m.

- Dam Structure: Gravity concrete dam.

  - Maximum height: 21.50 m.

  - Total spillway length: 62.0 m.

- Powerhouse:

  - Number of units: 02 units.

  - Turbine type: Horizontal Francis turbine.

  - Design head (Htt): 92.10 m.

### 3. INVESTMENT AND PROGRESS

- Total Investment Cost: 144 billion VND (including VAT).

- Implementation Schedule:

  - Total construction period: 30 months.

  - Commencement date: Quarter I, 2023 (expected).

  - Commercial Operation Date (COD): April 2025 (expected).

SUMMARY OF ECONOMIC & FINANCIAL INDICATORS

| Indicator | Economic (EIRR) | Financial (FIRR) |
| --- | --- | --- |
| Internal Rate of Return | 12.39% | 14.38% |
| Net Present Value (NPV) | 25.81 billion VND | 69.35 billion VND |
| B/C Ratio | 1.21 | 1.47 |