### DAK GLUN 3 HYDROPOWER PROJECT

Location: Tuy Duc District, Dak Nong Province Stage: Technical Design

#### 1. GENERAL INTRODUCTION (CHƯƠNG 1: GIỚI THIỆU CHUNG)

- Overview: Dak Glun 3 Hydropower Project is the 3rd stage in a 5-stage plan on the Dak Glun stream.

- Location: The main dam, auxiliary dam, and powerhouse are located in Quang Tam Commune, Tuy Duc District, Dak Nong Province.

- Project Objectives:

  - Main task: Power generation with an installed capacity of 7.1 MW and an average annual energy output of 23.83 million kWh.

  - Secondary task: Create a reservoir with an active storage capacity of approximately 121,000 m3.

  - Socio-economic impact: Create jobs for local residents and promote regional economic development.

#### 2. DESIGN SPECIFICATIONS (CẤP CÔNG TRÌNH & TẦN SUẤT THIẾT KẾ)

- Design Classification: Grade III.

- Flood Design Frequency (P): 1.5%.

- Flood Check Frequency (P): 0.5%.

- Power Generation Reliability (P): 85%

#### 3. MAIN TECHNICAL PARAMETERS (THÔNG SỐ KỸ THUẬT CHÍNH)

I. Reservoir (Hồ chứa):

- Normal Water Level (MNDBT): 500.00 m

- Dead Water Level (MNC): 498.50 m

- Total Capacity (Vtb): 389,000 m3

- Active Capacity (Vhi): 121,000 m3

II. Main Dam (Đập chính):

- Right bank dam: Homogeneous earth dam, Max height 14.6 m.

- Left bank dam: Gravity concrete dam, Max height 16.7 m.

- Spillway:

  - Free overflow spillway (no gate): Crest elevation +500.0 m.

  - Gated spillway: 3 gates, Crest elevation +493.0 m.

III. Power Generation Line (Tuyến năng lượng):

- Intake: Reinforced concrete, design flow 9.49 m3/s, crest elevation 496.50 m.

- Canal: Non-pressure, reinforced concrete, length L = 6,310 m, slope 1.5‰.

- Forebay (Bể áp lực): Length 57 m, design flow 9.49 m3/s.

- Powerhouse: Open-type powerhouse, 2 units, Francis horizontal turbines.

#### 4. INVESTMENT & COST (TỔNG DỰ TOÁN)

- Total Investment (after tax): 266,430,000,000 VND (Two hundred sixty-six billion, four hundred thirty million Vietnamese Dong).

- Breakdown:

  - Compensation, support, and resettlement: 3.5 billion VND.

  - Construction costs: ~168.6 billion VND.

  - Equipment costs: ~60.3 billion VND.

#### 5. LEGAL BASIS (CƠ SỞ PHÁP LÝ)

The project is established based on :

- Law on Construction No. 50/2014/QH13.

- Law on Water Resources No. 17/2012/QH13.

- Law on Environmental Protection No. 55/2014/QH13.

- Decree No. 06/2021/NĐ-CP on quality management and maintenance of construction works.

- Decree No. 10/2021/NĐ-CP on management of construction investment costs.